# ESP32 Print Room Display Documentation

Generated: 2026-02-07

## Overview

This build uses an ESP32 driving a 2.4"/2.8" ILI9341 SPI TFT display with an XPT2046 resistive touchscreen. The UI shows a two-page dashboard: a Printers page (Bambu 1 + Bambu 2, 6 cards) and an Air Quality page (3 large cards). Touch buttons along the bottom switch pages.

| Item | Notes |
| --- | --- |
| ESP32 DevKit (esp32dev) | ESP-IDF framework in ESPHome; static IP used for easy access. |
| ILI9341 SPI TFT module | Pins: VCC, GND, CS, RESET, DC, SDI(MOSI), SCK, LED, (SDO/MISO unused for TFT |
| XPT2046 resistive touch controller | Pins: T_CS, T_IRQ, T_CLK, T_DIN, T_DO. Shares SPI CLK/MOSI/MISO with TFT. |
| Dupont jumpers / Y-cables | Used to share SPI CLK and MOSI lines between TFT and touch without soldering. |

Hardware Used

).

## Power Notes (confirmed working)

VCC (module power): 5V  VCC is OK.

Backlight: 3.3V  LED (backlight) used in this build.

Ground: All grounds common (ESP32 GND  TFT GND  Touch GND).

## Wiring Table (Spreadsheet format)

| Signal | TFT Pin Label | Touch Pin Label | ESP32 GPIO / Powe | r Notes |
| --- | --- | --- | --- | --- |
| SPI CLK | SCK | T_CLK | GPIO18 | Y Cable |
| SPI MOSI | SDI(MOSI) | T_DIN | GPIO23 | Y Cable |
| SPI MISO |  |  | GPIO19 |  |
| TFT Chip Select | CS | - | GPIO14 | TFT CS only. |
| TFT DC | DC | - | GPIO27 |  |
| TFT Reset | RESET | - | GPIO4 |  |
| Touch Chip Select | - | T_CS | GPIO25 | Touch CS only. |
| Touch Interrupt | - | T_IRQ | GPIO26 | Pen IRQ line. |
| Power | VCC | - | 5V | 5V to VCC confirmed OK. |
| Backlight | LED | - | 3.3V | Backlight powered from 3.3V in thi |
| Ground | GND | GND | GND | Common ground. |

.

. u

s build.

Important: The TFT’s SDO(MISO) pin is not required for display-only operation. However, the touchscreen controller does

require its T_DO line to connect to the ESP32 MISO (GPIO19).

ESPHome / Display Settings

| Setting | Value |
| --- | --- |
| ESPHome name | remote-display-screen |
| Framework | ESP32 + ESP-IDF |
| Display driver | ili9xxx (model: ili9341) |
| Display rotation | 270° |
| Display palette | 8BIT |
| Display update interval | 2s |
| Touch driver | xpt2046 |
| Touch update interval | 50ms |
| Touch threshold | 400 |
| Touch calibration | x_min=340, x_max=3760, y_min=240, y_max=3760 |
| Touch transform | swap_xy: true, mirror_x: true, mirror_y: true |
| Static IP | 192.168.1.24 / 255.255.255.0, GW 192.168.1.254 ****optional or use your own |

Home Assistant Entities Used ******Replace with your own Entity IDs******

| Purpose | Entity ID |
| --- | --- |
| AQI | sensor.game_room_air_quality_s2_air_quality_index |
| eCO2 | sensor.game_room_air_quality_s2_eco2 |
| TVOC | sensor.game_room_air_quality_s2_tvoc |
| Bambu 1 status | sensor.bambu_p1s_1_print_status |
| Bambu 1 progress | sensor.bambu_p1s_1_print_progress |
| Bambu 1 remaining | sensor.bambu_p1s_1_remaining_time |
| Bambu 2 status | sensor.p1s_2_print_status |
| Bambu 2 progress | sensor.p1s_2_print_progress |
| Bambu 2 remaining | sensor.p1s_2_remaining_time |

Final ESPHome YAML

Full configuration:

File found in zip file attached

## Notes / Tips

- If touch feels sluggish, consider increasing the display update_interval (e.g., 3–5s) or reducing touch polling (e.g., 80–120ms).

- If touch coordinates are mirrored, adjust the transform section (swap_xy / mirror_x / mirror_y).

- If you ever move the screen rotation again, touch transform usually needs to change too.